|  | ## Critical Control Point ## (CCP) | Hazards | Critical Limits | Monitoring |  |  |  | Corrective Action(s) | Verification | Records |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | What | How | Frequency | Who |  |  |  |
| 21 | Vat Pasteurization (with continuous agitation) | M - Pathogens | The temperature as measured by the indicating thermometer must be at a minimum of 155°F. The holding time must be at a minimum of 30 minutes. The air space thermometer must indicate a minimum of 160°F. | Pasteurization complies with PMO. | Check and sign-off on recording charts. Record both air space and indicating temperatures. Mark beginning and end of holding time. Annotate the batch information for each batch on the recording chart including indicating vs. recording comparison. Notify QC if seal is broken so QC can notify State. | Every Batch Quarterly: State inspector reviews and signs-off charts, calibrates thermometers, time charts for accuracy and checks seals. | Pasteurizer operator for every batch made. | During Pasteurization: Continue pasteurizing until time/temperature criteria are met. Document all actions on recording chart. After Pasteurization: If the product is found not to have met the time/temperature criteria place all affected product on hold and evaluate to reprocess or destroy. | QC reviews and signs-offs, recording charts at least once a week. | Temperature/time charts, Calibration Records, Corrective Action Records. Location: XX . |